FACULTY OF ENGINEERING – CIVIL ENGINEERING PROGRAM

Environmental Management and Sustainable Development – 6th semester

FIRST PARTIAL EXAM (PRACTICAL, GROUP WORK)

| Names of group members (maximum 4) | Signature | ID number |
| --- | --- | --- |
| 1. |  |  |
| 2. |  |  |
| 3. |  |  |
| 4. |  |  |

| Date | Time | Total points | Grade |
| --- | --- | --- | --- |
|  | 120 minutes | 100 points |  |

## Instructions

- Work in groups of maximum 4 students. Each group hands in one copy with the names of all members.

- You do not need a calculator. You use the Leopold matrix only to identify which part of the environment each impact affects, if the impact is good or bad (+ / −), and how strong it is (High, Medium, Low). You do not calculate magnitude, importance or sums.

- Use the ISO 14001:2015 words. If an answer mixes up aspect and impact, or impact and component, it gets zero points.

- You cannot use cell phones or any other material.

- Read Part II before you fill in any matrix.

| Part | Content | Points | Suggested time |
| --- | --- | --- | --- |
| I | ISO 14001 concepts: aspect, impact and environmental performance | 25 | 25 min |
| II | Guide to fill in the Leopold matrix (reading) | — | 5 min |
| III | Practical cases: identify effects with the Leopold matrix | 50 | 35 min |
| IV | From impact to environmental performance | 25 | 25 min |
|  | Total | 100 | 90 min |

# Part I. ISO 14001 concepts (25 points)

Question 1 (9 points). In your own words, and based on ISO 14001:2015, define each term and give your own example from a civil works project.

| Term | Definition (in your own words) | Example in civil works |
| --- | --- | --- |
| Environmental aspect |  |  |
| Environmental impact |  |  |
| Environmental performance |  |  |

Question 2 (8 points). Classify each sentence as Aspect (A), Impact (I), Performance (P) or None (N). For the ones you mark N, explain why in one line.

| No. | Sentence | A / I / P / N | Explanation (only N) |
| --- | --- | --- | --- |
| a | Generation of construction and demolition waste (CDW) when a house is demolished |  |  |
| b | Sediment builds up in the stream bed downstream of the work site |  |  |
| c | 95 % of the CDW of the month went to an authorized waste manager (target: 90 %) |  |  |
| d | Emission of dust (particulate matter) from dump trucks on an unpaved road |  |  |
| e | Breathing problems in people of the nearby neighborhood |  |  |
| f | Purchase of a new hydraulic hammer for the site |  |  |
| g | Lower water use per m³ of concrete than in the last quarter |  |  |
| h | Loss of vegetation cover in the area where topsoil is removed |  |  |

Question 3 (8 points). Complete the chain for the activity “pouring concrete and washing concrete mixer trucks at the work site”. Write the indicator as a formula with its unit (you do not need to calculate it).

| Activity | Environmental aspect | Environmental impact | Affected component |
| --- | --- | --- | --- |
| Pouring concrete and washing mixer trucks |  |  |  |

# Part II. Guide to fill in the Leopold matrix (you must read this)

In this exam you use the Leopold matrix to identify which impact affects which component of the environment, and how strong the effect is. You do not give numbers.

Structure. The columns are the project activities (the actions that create environmental aspects). The rows are the components of the environment (what receives the impact). Fill in a cell only if the activity causes an impact on that component.

Components of the environment:

| Medium | Component |
| --- | --- |
| Abiotic | Atmosphere – air quality |
| Abiotic | Atmosphere – noise and vibration |
| Abiotic | Surface water |
| Abiotic | Groundwater |
| Abiotic | Soil |
| Biotic | Plants and vegetation cover |
| Biotic | Wildlife (fauna) |
| Perceptual | Landscape |
| Socioeconomic | Population (health and safety) |
| Socioeconomic | Economy and jobs |
| Socioeconomic | Mobility and access |

How to fill in each cell: impact code + type + level. Example: I-5 / − / H means that impact I-5 is negative and has a high effect on that component. If there is more than one impact in the same cell, write them one below the other.

| Symbol | Meaning | Reference criterion |
| --- | --- | --- |
| + | Positive impact | Makes the component better than the situation without the project |
| − | Negative impact | Makes the component worse than the situation without the project |
| H | High effect | Large, strong, hard to reverse, or on a protected or sensitive component |
| M | Medium effect | Local, reversible in months, with known control measures |
| L | Low effect | Small area, temporary, reversible in days or weeks |
| (empty) | No interaction | The activity does not affect that component |

Significance rule (ISO 14001, clause 6.1.2). An environmental aspect is significant if it causes at least one negative impact of level H, or if there is a legal requirement for the affected component. Significant aspects go to Part IV.

# Part III. Practical cases with the Leopold matrix (50 points)

Each case practices a different skill: place given impacts (A), identify impacts from zero (B), check a matrix with errors (C) and compare project phases (D).

## Case A. Upgrade of a rural road, 8.4 km long (15 points)

A 6 m wide gravel road will be paved in a rural hillside area. It crosses two streams and passes next to 3 ha of secondary forest. Three rural communities, with 420 people, use the road to take coffee and plantain to market. The assessment team already found these impacts:

| Code | Identified impact | Affected component | Medium |
| --- | --- | --- | --- |
| I-1 | Worse air quality from dust (particulate matter) |  |  |
| I-2 | Higher noise levels |  |  |
| I-3 | Loss of vegetation cover |  |  |
| I-4 | Erosion and less stable soil on slopes |  |  |
| I-5 | More sediment and turbidity in the streams |  |  |
| I-6 | Wildlife moves away and its habitat is divided |  |  |
| I-7 | Local jobs |  |  |
| I-8 | Better access to markets and shorter travel times |  |  |
| I-9 | Temporary traffic problems because of road closures |  |  |
| I-10 | Risk of animals being hit by vehicles |  |  |
| I-11 | Runoff control and less erosion |  |  |
| I-12 | Worse visual quality of the landscape because of cuts |  |  |
| I-13 | Less dust in the air because the road is no longer unpaved |  |  |

- (4 pts) In the table above, write the affected component and the medium (abiotic, biotic, perceptual or socioeconomic) of each impact.

- (6 pts) Fill in the matrix: in each cell where there is an interaction, write code + type + level (for example, I-5 / − / H).

| Component \ Activity | A1 Clearing and topsoil removal | A2 Cuts and fills | A3 Transport of materials | A4 Drainage works | A5 Paving | A6 Road operation |
| --- | --- | --- | --- | --- | --- | --- |
| Atmosphere – air quality |  |  |  |  |  |  |
| Atmosphere – noise and vibration |  |  |  |  |  |  |
| Surface water |  |  |  |  |  |  |
| Groundwater |  |  |  |  |  |  |
| Soil |  |  |  |  |  |  |
| Plants and vegetation cover |  |  |  |  |  |  |
| Wildlife (fauna) |  |  |  |  |  |  |
| Landscape |  |  |  |  |  |  |
| Economy and jobs |  |  |  |  |  |  |
| Mobility and access |  |  |  |  |  |  |

- (3 pts) Answer by counting cells, with no math: Which component receives the most negative impacts? Which component receives the most level H impacts? Are they the same? Explain why frequency is not the same as severity.

- (2 pts) Write the environmental aspect that causes two of the impacts you marked − / H.

## Case B. 12-story apartment building in a city (15 points)

A tower with 96 apartments and 2 basements, on a 2,400 m² lot next to a school and in front of a storm water channel. The excavation is 14,000 m³ and the water table is at 4 m, so water must be pumped out during the excavation. The structure is cast-in-place concrete and the facade is masonry.

- (6 pts) Identify at least 7 impacts of the project. For each one, write the activity, the aspect that causes it and the affected component.

| Code | Activity | Environmental aspect | Environmental impact | Affected component |
| --- | --- | --- | --- | --- |
| B-1 |  |  |  |  |
| B-2 |  |  |  |  |
| B-3 |  |  |  |  |
| B-4 |  |  |  |  |
| B-5 |  |  |  |  |
| B-6 |  |  |  |  |
| B-7 |  |  |  |  |

- (6 pts) Fill in the matrix with your impacts (code + type + level). Include at least one impact from the operation phase.

| Component \ Activity | Demolition and topsoil removal | Basement excavation | Pumping groundwater | Transport of material | Structure and facade | Building operation |
| --- | --- | --- | --- | --- | --- | --- |
| Atmosphere – air quality |  |  |  |  |  |  |
| Atmosphere – noise and vibration |  |  |  |  |  |  |
| Surface water |  |  |  |  |  |  |
| Groundwater |  |  |  |  |  |  |
| Soil |  |  |  |  |  |  |
| Plants and vegetation cover |  |  |  |  |  |  |
| Wildlife (fauna) |  |  |  |  |  |  |
| Landscape |  |  |  |  |  |  |
| Population (health and safety) |  |  |  |  |  |  |
| Economy and jobs |  |  |  |  |  |  |
| Mobility and access |  |  |  |  |  |  |

- (3 pts) Which element near this lot (school, channel, neighbors) makes an impact stronger, when in another place it would be low? Give an example and name one legal requirement that applies (noise, wastewater discharge or CDW).

## Case C. Check the matrix of a wastewater treatment plant (WWTP) (10 points)

A wastewater treatment plant (WWTP) for 25,000 people, 400 m from houses. Today the town discharges raw wastewater into the river, and farmers downstream use the river water for irrigation. A consultant delivered this matrix. It has at least 6 errors in concepts.

| Component \ Activity | C1 Earthworks | C2 Construction of reactors | C3 Sludge disposal | C4 Discharge of treated effluent | C5 Generation of odors |
| --- | --- | --- | --- | --- | --- |
| Surface water | − / L |  | − / M | − / H | − / M |
| Atmosphere – air quality | − / L | − / L | − / M |  |  |
| Soil | − / M | − / L | + / H |  |  |
| Soil pollution from sludge |  |  | − / M |  |  |
| Wildlife (fauna) |  | + / M |  |  |  |
| Population (health and safety) |  |  |  |  |  |

- (10 pts) Find 6 errors. For each one, write the row, column or cell, why it is an error and how you correct it.

| No. | Location | Why it is an error | Correction |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |

## Case D. 120 m vehicle bridge over a river (10 points)

A prestressed concrete bridge with 2 piers in the river bed. It replaces a river ferry and cuts the trip between two towns from 45 to 12 minutes. To build the piers, temporary cofferdams must stay in the river for 7 months, in an area where fish lay their eggs.

- (5 pts) Fill in the matrix with type and level (+/− and H/M/L). Include at least 2 positive impacts.

| Component \ Activity | Cofferdams and pier foundations | Placing beams and deck | Vehicle traffic (operation) | Maintenance (operation) |
| --- | --- | --- | --- | --- |
| Surface water |  |  |  |  |
| Atmosphere – noise and vibration |  |  |  |  |
| Wildlife (fish) |  |  |  |  |
| Landscape |  |  |  |  |
| Mobility and access |  |  |  |  |
| Economy and jobs |  |  |  |  |

- (5 pts) Compare the construction phase with the operation phase: which components change from negative to positive or the other way? Name one limitation of the Leopold matrix that this case shows (time, accumulation, synergy or subjectivity) and how you would complement it.

# Part IV. From impact to environmental performance (25 points)

The matrix identifies impacts; ISO 14001 also asks you to plan actions and measure if they work (clauses 6.2, 8.1 and 9.1). Work with the significant aspects of one of the cases, A or B.

Question 4 (12 points). For 3 significant aspects, complete the table. The objectives must be measurable and have a deadline.

| Significant aspect | Impact and component | Environmental objective | Operational control | Indicator (formula and unit) | Target and deadline |
| --- | --- | --- | --- | --- | --- |

Question 5 (6 points). A construction company reports these data. Calculate in your head the normalized indicator (t of CDW per m³ of concrete, and m³ of water per m³ of concrete) and say if the environmental performance improved. Explain why looking only at the absolute numbers gives a wrong conclusion.

| Month | CDW generated (t) | Concrete poured (m³) | Water used (m³) | CDW / concrete (t/m³) | Water / concrete (m³/m³) |
| --- | --- | --- | --- | --- | --- |
| March | 120 | 1,000 | 900 |  |  |
| April | 160 | 2,000 | 1,200 |  |  |

Question 6 (7 points). A site supervisor says: “The project has good environmental performance because it has an environmental license, an approved Leopold matrix and an Environmental Management Plan with 300 pages.”

- (4 pts) Is this statement correct according to the ISO 14001 definition of environmental performance? Explain.

- (3 pts) Propose 3 objective pieces of evidence that would really show environmental performance in an audit (clause 9.1).

Grading rubric

Clear concepts count in the whole exam: if you mix up aspect, impact and component, you lose the point even if the matrix is full.

| Criterion | Excellent (100 %) | Acceptable (60 %) | Not enough (0–30 %) |
| --- | --- | --- | --- |
| Aspect / impact / performance | Uses the three terms with no confusion in the whole exam | One or two separate mistakes | Always confuses aspect with impact |
| Matrix structure | Activities in columns and components in rows; no impact or aspect appears as a row or column | One component or one activity is badly written | Mixes activities, components and impacts |
| Impact → component | Each impact is in the component that really receives it | One or two impacts in the wrong component | Random placement |
| Type and level | Correct sign compared with the situation without the project; level fits the site and its sensitive receptors | Reasonable levels with some mistakes | Wrong signs or levels with no link to the site |
| Performance indicators | Normalized, with formula, unit, target and deadline | Missing target or deadline | Indicators you cannot measure (“improve management”) |
| Critical thinking | Separates frequency from severity and knows the limits of Leopold | Mentions limits but does not explain them | Uses the matrix as a final decision on the project |